# Atlas 1.0-6.0 Complete Engineering Log

Consolidated English Edition for GitHub / PBL Portfolio

Project: Atlas AI Mentor / Hardware Robot Series

Recorder: Eric

Scope: Smart Plant Care System 1.1; Atlas 1.0, 2.0, 3.0, 4.0, 5.0, and 6.0

English edition compiled: 2026-09-14

# Editorial Note

This is a consolidated English engineering-log edition. Atlas 1.0-5.0 is translated and reorganized from the original moderate-detail engineering log. Atlas 6.0 is added from the project engineering log, development plan, firmware revisions, control-test files, vision/dataset/model-release scripts, final integration programs, and archived project/chat records. Exact dates are used only where project evidence contains them; later records with no reliable single date are labeled by development period rather than inventing a date.

Failures, wrong assumptions, abandoned routes, and debugging decisions are deliberately preserved because they are part of the engineering evidence.

# Version Evolution Overview

| Version / Project | Core Goal | Main Capabilities | Evidence / Outputs |
| --- | --- | --- | --- |
| Smart Plant Care System 1.1 | AI + hardware plant-care prototype | Arduino, sensors, automatic lighting/watering, iterative hardware debugging | Project log, physical prototype, demo evidence |
| Atlas 1.0 | AI Research Mentor Robot foundation | Project Log, mentor advice, research support, basic memory | project_log.txt, memory.json, mentor-response records |
| Atlas 2.0 | Project Management Robot | Project Database, Daily Task, Bug Manager, Weekly Report | atlas2_data.json, weekly_report.txt, bug records |
| Atlas 3.0 | Eric Digital Twin Mentor | Profile, Skill, History, Planner, Emotion, Recommendation | atlas3_data.json, learning plan, mentor recommendations |
| Atlas 4.0 | Multimodal Mentor Robot | Vision, voice input/output, memory, proactive mentor, hardware feedback | atlas4_full_main.py, multimodal logs |
| Atlas 5.0 | Stable Hardware Robot Edition | ESP32, OLED, pan/tilt, LEDs, voice, camera, Safe Mode | Final Python program, JSON/TXT logs, screenshots, demo video |
| Atlas 6.0 | Mobile autonomous hardware robot | L298N drive, dual encoders, closed-loop motion, US-100 sensing, Wi-Fi/TCP safety, C950 AI vision, autonomous recovery, reintegrated expression/voice | ESP32 integrated firmware, Python vision/master programs, model-release scripts, test logs, demo video |

# Chronological Engineering Log

# 1. 2026-07-01 - Smart Plant Care System 1.1 + Atlas 1.0 AI Research Mentor Robot

Recorder: Eric    Phase: Code Debugging / Test Optimization

## 1. Tasks and Completion

Planned tasks

Complete the stage record for Smart Plant Care System 1.1 and preserve it as the hardware-learning foundation for Atlas.

Add automatic Project Log generation to Atlas 1.0.

Test whether Atlas can read the latest project log and provide mentor-style engineering advice.

Actual completed work

Completed the Smart Plant Care System 1.1 record and linked it to later Atlas hardware learning.

Upgraded main.py so Atlas can record goals, completed work, problems, solutions, next steps, and mentor advice.

Saved logs to both memory.json and project_log.txt.

Validated Mentor Advice and Research Support responses based on recent engineering records.

## 2. Tools, Materials, and Software

Software: Python, main.py, JSON, memory.json, project_log.txt, terminal I/O. Hardware: plant-care prototype, Arduino sensors, soil-moisture and temperature/humidity sensors, PC.

## 3. Process Record - Key Operations / Parameters / Steps

Formalized the plant-care project as prior experience in sensor reading, wiring, and iterative debugging.

Created a fixed Project Log schema so later portfolio evidence would not become fragmented.

Used realistic prompts about completed work and repeated failures to test whether Atlas could respond as an engineering mentor rather than a generic chatbot.

## 4. Problems and Observed Phenomena

Log storage location was initially unclear, creating a risk that Atlas would not recall previous work.

Soil-related raw sensor data remained unstable.

Long debug sessions created a tendency to change several modules at once.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

A single storage path was not robust enough for memory retrieval.

Sensor instability could come from hardware quality, wiring, sampling range, or insufficient environmental variation.

Unrecorded failures would cause repeated troubleshooting.

Attempts

Write the same structured log into both memory.json and project_log.txt.

Keep failed tests as evidence instead of deleting them.

Reduce the next task to one testable question at a time.

Final solution / decision: Atlas 1.0 adopted a fixed-field Project Log, dual-file storage, and mentor advice written back into the log. Unstable plant sensors were deliberately left as a separate hardware-debug task instead of expanding scope.

## 6. Test / Experimental Results

Automatic Project Log writing worked.

Atlas generated mentor advice from the latest log.

Atlas 1.0 became a basic AI mentor with memory, logging, mentor advice, and research-support behavior.

## 7. Remaining Issues

Plant sensor reliability still required separate testing.

Cross-day recall still needed validation.

## 8. Next Work Plan

Test recall of the previous day's Project Log.

Prepare the transition to Atlas 2.0 project-management functions.

# 2. 2026-07-02 (Round 1) - Atlas 2.0 Project Management Robot

Recorder: Eric    Phase: Code Debugging / Test Optimization / Reporting

## 1. Tasks and Completion

Planned tasks

Upgrade Atlas from a logging robot into a project-management robot.

Implement Project Database, Daily Task, Bug Manager, and Weekly Report.

Write tasks, bugs, and reports into project_log.txt for traceable evidence.

Actual completed work

Tested the Project Database with Smart Plant Care, Atlas 1.0, and Atlas 2.0 entries.

Created and reviewed a Daily Task.

Recorded and resolved an OpenCV face-detection bug in Bug Manager.

Generated a Weekly Report summarizing projects, tasks, bugs, and mentor judgments.

## 2. Tools, Materials, and Software

Software: Python, atlas2_main.py and staged scripts, JSON files, project_log.txt, OpenCV. Hardware: PC and camera for vision-bug testing.

## 3. Process Record - Key Operations / Parameters / Steps

Validated Project Database log writing and project updates.

Created task fields for date, project, plan, estimated time, priority, reason, and status.

Used Bug Manager to document a real false-detection problem and its fix.

Generated a Weekly Report to test the complete management loop.

## 4. Problems and Observed Phenomena

An accidental terminal input polluted a project status field.

Early OpenCV person detection remained active after Eric left the frame.

It was necessary to verify that the Weekly Report actually aggregated all data sources.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Terminal input validation was weak.

Person detection was sensitive to background, range, and lighting.

Module data structures were still distributed.

Attempts

Corrected the project status and progress back to completed / 100%.

Switched the unstable detection direction toward face detection.

Repeated logging tests for all four management modules.

Final solution / decision: Kept the mistaken input as authentic debug evidence, corrected the project record afterward, changed the vision test to face detection, and used the Weekly Report as the stage-closing artifact.

## 6. Test / Experimental Results

All four Atlas 2.0 management functions ran successfully.

Daily Task and review worked.

Bug Manager supported add/search/update/summary operations.

Weekly Report produced project, task, bug, and next-week summaries.

## 7. Remaining Issues

Project Database input validation could be stronger.

Chat-history writing into project_log still needed testing.

## 8. Next Work Plan

Merge the separated Atlas 2.0 modules into one main program.

Begin Atlas 3.0 profile and long-term growth modeling.

# 3. 2026-07-02 (Round 2) - Atlas 2.0 Integrated Build and Stage Closeout

Recorder: Eric    Phase: Code Debugging / Test Optimization / Reporting

## 1. Tasks and Completion

Planned tasks

Merge Project Database, Daily Task, Bug Manager, and Weekly Report.

Add a Demo-video task and regenerate the Weekly Report.

Test data migration and consolidated logging.

Actual completed work

Created atlas2_data.json and migrated data from the earlier separated JSON files.

Ran the integrated atlas2_main.py one-click overview.

Added an Atlas 2.0 demo-video Daily Task.

Recorded the unresolved project_log chat-record issue as a Bug.

Generated a corrected Weekly Report showing Atlas 2.0 at 100% completion.

## 2. Tools, Materials, and Software

Software: Python, atlas2_main.py, atlas2_data.json, project_log.txt, weekly_report.txt. Hardware: PC.

## 3. Process Record - Key Operations / Parameters / Steps

Migrated legacy project/task/bug/report records before launching the integrated program.

Used the one-click overview as a stage-level demonstration.

Preserved the unresolved chat-log issue inside Bug Manager instead of hiding it.

## 4. Problems and Observed Phenomena

Multiple JSON files were difficult to maintain.

Chat content was still not being written into project_log.

Demo filming was only partially complete.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

The project grew module-by-module, producing fragmented files.

Chat logging and normal project logging did not share one write path.

Demo filming required synchronized software, hardware, and narration.

Attempts

Use a unified main program and consolidated JSON.

Keep unresolved cross-version issues visible in Bug Manager.

Regenerate reports after correcting bad records.

Final solution / decision: Atlas 2.0 was accepted as functionally complete. The remaining chat-log issue was preserved as a known cross-version limitation rather than blocking the core project-management functions.

## 6. Test / Experimental Results

Atlas 2.0 progress was corrected to 100%.

The integrated demo view could show projects, tasks, bugs, and reports.

The Weekly Report retained real bug-fix evidence.

## 7. Remaining Issues

Chat-history logging remained unresolved.

Demo video still required completion.

## 8. Next Work Plan

Start Atlas 3.0 Profile, Skill Database, and Project History.

Carry Atlas 2.0 management capability into the personalized mentor layer.

# 4. 2026-07-03 (Round 1) - Atlas 3.0 Digital Twin Mentor Robot - Profile / Skill / History

Recorder: Eric    Phase: System Design / Code Debugging / Test Optimization

## 1. Tasks and Completion

Planned tasks

Build an Eric Profile with age, goals, projects, interests, strengths, and weaknesses.

Create a Skill Database that can influence recommendations.

Create Project History linking the plant system and Atlas 1.0-3.0.

Actual completed work

Completed Eric Profile and identity-aware responses.

Created Skill Database values including Arduino 95, Python 80, OpenCV 75, YOLO 60, and ROS2 0.

Atlas could recommend ROS2 instead of repeating basic Arduino.

Completed Project History explaining how earlier projects contributed skills to later versions.

## 2. Tools, Materials, and Software

Software: Python, profile.json, skills.json, history.json, project_log.txt. Hardware: PC.

## 3. Process Record - Key Operations / Parameters / Steps

Locked Eric's long-term target as AI Systems Engineer and project-based learning as the preferred learning style.

Used scored skills so mentor recommendations were data-driven.

Tested project-history search and cross-project skill transfer explanations.

## 4. Problems and Observed Phenomena

Generic greetings were not sufficiently personalized.

Exact keyword matching failed on some near-synonyms.

ROS2 was clearly identified as the largest skill gap.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Without Profile, Skill, and History, recommendations would remain generic.

The current search used simple lexical matching rather than semantic retrieval.

Eric already had enough Arduino/Python/OpenCV experience to justify a systems-level next step.

Attempts

Test each database independently with fixed questions.

Keep semantic-search improvement as a later enhancement.

Use the data chain to justify the ROS2 recommendation.

Final solution / decision: Atlas 3.0 shifted from feature accumulation to a data-driven Eric Digital Twin: first know who Eric is, what he knows, and what he has built; then recommend the next learning step.

## 6. Test / Experimental Results

Eric Profile worked.

Skill Database worked and produced a ROS2 recommendation.

Project History explained skill transfer from the plant project into Atlas.

## 7. Remaining Issues

Near-synonym project search still needed improvement.

ROS2 had been identified but not yet learned.

## 8. Next Work Plan

Continue with Learning Planner, Emotion Memory, and Mentor Recommendation.

# 5. 2026-07-03 (Round 2) - Atlas 3.0 Digital Twin Mentor Robot - Planner / Emotion / Recommendation / Data Integration

Recorder: Eric    Phase: Code Debugging / Test Optimization / Reporting

## 1. Tasks and Completion

Planned tasks

Generate a daily learning plan.

Record engineering emotion/debug state.

Generate mentor recommendations from Profile + Skill + History + Planner + Emotion.

Integrate Atlas 1.0-3.0 data.

Actual completed work

Built a ROS2-oriented Learning Planner.

Recorded a real Arduino sensor failure inside Emotion Memory.

Generated a recommendation explaining why ROS2 was the next priority.

Created atlas3_data.json and additional integrated/unified JSON files.

## 2. Tools, Materials, and Software

Software: Python, learning_plan.json, emotion_memory.json, mentor_recommendation.json, atlas3_data.json, atlas_unified_data.json. Hardware: PC; Arduino sensor hardware referenced in the debug record.

## 3. Process Record - Key Operations / Parameters / Steps

Read Profile, Skill Database, and Project History in sequence.

Generated a plan and exposed the reasoning behind it.

Logged frustration as engineering-state context, not as a psychological diagnosis.

Migrated several JSON sources into unified structures.

## 4. Problems and Observed Phenomena

Soil/temperature-humidity sensor data was still unstable.

Fragmented JSON files could make recommendations incomplete.

The recommendation system needed evidence that results were not random.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Hardware quality/wiring/power could explain sensor instability.

Fragmented context weakened recommendation quality.

Recommendations needed a transparent Profile -> Skill -> History -> Plan -> Emotion reasoning chain.

Attempts

Record hardware failure as Emotion Memory.

Display recommendation logic.

Consolidate data sources.

Final solution / decision: Completed a six-part Atlas 3.0 loop: Profile, Skill, History, Planner, Emotion, Recommendation, integrated into unified data structures.

## 6. Test / Experimental Results

Digital Twin foundation completed.

Atlas could explain why ROS2 was recommended.

Cross-version data migration became possible.

## 7. Remaining Issues

ROS2 learning plan had not yet been executed.

Hardware sensor issues remained separate.

Vision, voice, and hardware feedback still needed Atlas 4.0 integration.

## 8. Next Work Plan

Enter Atlas 4.0 multimodal development.

# 6. 2026-07-04 (Round 1) - Atlas 4.0 Multimodal Mentor Robot - Proactive Mentor / Voice / Hardware Feedback

Recorder: Eric    Phase: Hardware Integration / Code Debugging / Test Optimization

## 1. Tasks and Completion

Planned tasks

Add a proactive Morning Brief based on long-term memory.

Test voice output.

Test physical hardware feedback through serial commands.

Actual completed work

Completed Proactive Mentor using atlas_unified_data.json.

Voice output successfully spoke greetings, Morning Briefs, and custom text.

Established a working COM4 hardware connection after serial debugging.

Tested STATUS, HAPPY, WARNING, and NOD commands.

## 2. Tools, Materials, and Software

Software: Python, pyttsx3, pyserial, atlas4_proactive_mentor.py, atlas4_hardware_feedback.py, atlas4_full_main.py. Hardware: Arduino/CH340 serial board, LEDs, OLED, servo, USB cable, PC.

## 3. Process Record - Key Operations / Parameters / Steps

Generated Morning Brief from long-term data.

Tested TTS independently.

Tried COM4/COM6, documented FileNotFoundError/PermissionError, then closed competing serial tools.

Used PING/PONG before behavior commands.

## 4. Problems and Observed Phenomena

COM4/COM6 sometimes failed to open.

PermissionError indicated another process owned the port.

Sometimes no serial device was detected.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Arduino IDE Serial Monitor, plotter, or another Python process could own the port.

Configured COM port could differ from the real device.

USB cable, drivers, and reconnect state affected detection.

Attempts

List ports first.

Close any serial monitor.

Confirm PING/PONG before behavior testing.

Final solution / decision: Established a repeatable serial-debug sequence: detect device -> close competing process -> PING/PONG -> send hardware commands. This pattern was reused in Atlas 5.0.

## 6. Test / Experimental Results

Proactive Mentor generated Morning Briefs.

Voice Output worked.

Hardware feedback could be controlled through COM4.

Atlas 4.0 formed a memory -> proactive advice -> voice -> hardware-feedback chain.

## 7. Remaining Issues

Hardware still depended on stable serial access.

Vision, voice input/output, memory, and hardware still needed one integrated main program.

## 8. Next Work Plan

Build Atlas 4.0 Full Main and the Atlas 1.0-4.0 integrated program.

# 7. 2026-07-04 (Round 2) - Atlas 4.0 Full Main + Atlas 1.0-4.0 Integrated Program

Recorder: Eric    Phase: Code Debugging / Test Optimization / Reporting

## 1. Tasks and Completion

Planned tasks

Integrate Vision, Voice Input, Voice Output, Memory, Proactive Mentor, and Hardware Feedback.

Combine major capabilities from Atlas 1.0-4.0.

Validate vision, voice, and overall status reporting.

Actual completed work

Ran Atlas 4.0 Full status overview.

Validated vision frame counts with Eric present/absent.

Completed repeated voice-output tests.

Launched the Atlas 1.0-4.0 master program and displayed version capabilities and data statistics.

## 2. Tools, Materials, and Software

Software: Python, OpenCV, pyttsx3, pyserial, atlas4_full_main.py, atlas_all_versions_main.py, atlas_unified_data.json. Hardware: USB camera, hardware-feedback controller, PC.

## 3. Process Record - Key Operations / Parameters / Steps

Checked dependency state and database/config paths.

Ran vision and voice separately before the all-version wrapper.

Kept camera and recording optional in the main demo to reduce simultaneous failure points.

## 4. Problems and Observed Phenomena

Vision depended on reliable camera opening.

Hardware feedback still risked serial-port conflicts.

Older version data formats differed.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Multimodal integration increased the number of possible failure sources.

Legacy data required normalization.

COM port changes remained an external dependency.

Attempts

Integrate modules in stages.

Use a unified menu for 1.0-4.0 capabilities.

Keep a text-only overview path that does not force all peripherals online.

Final solution / decision: Created an Atlas 1.0-4.0 software backbone with a unified menu and optional multimodal tests.

## 6. Test / Experimental Results

Atlas 4.0 multimodal main program was basically complete.

Atlas 1.0-4.0 overview and statistics worked.

The software architecture was ready for Atlas 5.0 physical-system engineering.

## 7. Remaining Issues

Physical hardware stability still depended on the controller/serial link.

Atlas 5.0 needed a hardware architecture rather than another software-only feature layer.

## 8. Next Work Plan

Archive 1.0-4.0 evidence and begin Atlas 5.0 Hardware Robot Edition.

# 8. 2026-07-05 - Atlas 5.0 Hardware Robot Edition - Architecture and Engineering Documentation

Recorder: Eric    Phase: System Design / Documentation

## 1. Tasks and Completion

Planned tasks

Redefine 5.0 as a hardware-engineering system rather than a repeat of 4.0 features.

Design the PC/controller/servo/OLED/LED/power/common-ground architecture.

Define Serial Protocol v5, behavior states, and file structure.

Actual completed work

Confirmed the revised Atlas 5.0 plan.

Defined IDLE, LISTENING, THINKING, SUCCESS, ENCOURAGE, WARNING, and ERROR.

Mapped each state to LEDs, OLED, head motion, voice style, and logging.

Prepared an engineering folder model: docs, firmware, python, data, logs.

## 2. Tools, Materials, and Software

Software/documentation: Word, Markdown, Python project-layout draft, Serial Protocol v5 draft. Planned hardware: ESP32/Arduino, pan/tilt servos, OLED, LEDs, external power, common GND.

## 3. Process Record - Key Operations / Parameters / Steps

Designed the system before adding new wiring.

Changed hardware control from isolated commands to state-machine orchestration.

Defined observable behavior for each state.

Created documentation and file-structure standards.

## 4. Problems and Observed Phenomena

Early plans risked turning 5.0 into another pile of 4.0 features.

Without a state machine, LED/OLED/servo behavior would remain fragmented.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Atlas 4.0 already demonstrated multimodal software; 5.0 needed physical reliability.

Behavior needed a common abstraction layer.

Attempts

Define state table first.

Define protocol and file structure before hardware integration.

Convert earlier logs into formal engineering documentation.

Final solution / decision: Locked the design principle: PC/Python handles voice, camera, and logic; ESP32 handles OLED/LED/servo behavior; both communicate through serial commands and READY_FOR_NEXT_COMMAND.

## 6. Test / Experimental Results

Architecture direction became explicit.

State-machine foundation was complete.

Engineering standards for later integration were established.

## 7. Remaining Issues

ESP32 head hardware had not yet been integrated.

Serial Protocol v5 still required real testing.

## 8. Next Work Plan

Build the pan/tilt head and basic ESP32 control.

# 9. 2026-07-06 - Atlas 5.0 Stage 2 - Two-Axis Pan/Tilt Head and ESP32 Basic Control

Recorder: Eric    Phase: Hardware Build / Code Debugging / Test Optimization

## 1. Tasks and Completion

Planned tasks

Build the pan/tilt head.

Use ESP32 instead of Arduino Uno as the main 5.0 controller.

Test PING, LEDTEST, ARM, CENTER, LEFT, RIGHT, UP, DOWN, NOD, SHAKE, and TEST.

Actual completed work

Completed the two-axis head; OLED was physically mounted as the face.

Locked an initial pin plan: pan GPIO18, tilt GPIO19, green GPIO25, yellow GPIO26, red GPIO27.

Powered servos from an independent 4xAA holder while ESP32 remained USB powered.

Validated basic LED and servo commands.

## 2. Tools, Materials, and Software

Software: Arduino IDE, ESP32Servo, Serial Monitor, Stage 2 firmware. Hardware: ESP32, two small servos, pan/tilt structure, LEDs, 4xAA holder, jumper wires, OLED.

## 3. Process Record - Key Operations / Parameters / Steps

Moved from the Arduino Uno direction to ESP32.

Powered servo red wires from the battery holder and tied battery negative to ESP32 GND.

Used ARM/DISARM around head actions.

Tested each motion command separately.

## 4. Problems and Observed Phenomena

Uno-based power/connection risk was undesirable.

Tilt servo initially had mechanical-direction/horn issues.

Servo current could not reliably come from ESP32 3V3/5V.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Servo startup current was high.

Mechanical neutral and software 90-degree center were not necessarily aligned.

Tilt axis carried more load.

Attempts

Reinstall/calibrate the tilt servo.

Use an independent 4xAA servo supply.

Validate each primitive command independently.

Final solution / decision: Adopted ESP32 + independent servo power + common ground + ARM/DISARM safety. Stage 2 intentionally stayed focused on basic motion rather than voice/camera/OLED integration.

## 6. Test / Experimental Results

Pan/tilt basic control passed.

Left/right/up/down/nod/shake motions worked.

The hardware base for later behavior firmware was established.

## 7. Remaining Issues

Power and cable routing needed mechanical stabilization.

Primitive actions still needed state-level behavior logic.

## 8. Next Work Plan

Enter Stage 3 power/cable engineering and Stage 4 behavior firmware.

# 10. 2026-07-07 - Atlas 5.0 Stage 3-4 - Power/Cable Engineering and Behavior Firmware v2

Recorder: Eric    Phase: Hardware Build / Code Debugging / Test Optimization

## 1. Tasks and Completion

Planned tasks

Fix the battery holder and critical cables.

Implement stable IDLE/LISTENING/THINKING/SUCCESS/ENCOURAGE/WARNING/ERROR behavior firmware.

Add READY_FOR_NEXT_COMMAND handshake.

Actual completed work

Completed battery and cable securing, wire labels, and U-shaped slack around the moving head.

Passed TEST20/TEST50 stability checks.

Completed Behavior Firmware v2 Stable with START, DONE, and READY_FOR_NEXT_COMMAND responses.

Validated all main behavior states and BEHAVIOR_TEST.

## 2. Tools, Materials, and Software

Software: Arduino IDE, ESP32Servo, atlas5_stage4_behavior_firmware_v2_stable.ino, Serial Monitor. Hardware: ESP32, pan/tilt servos, LEDs, 4xAA holder, cable labels and mounting materials.

## 3. Process Record - Key Operations / Parameters / Steps

Stabilized hardware before adding software complexity.

Uploaded behavior firmware and sent one state command at a time.

Waited for READY_FOR_NEXT_COMMAND before sending the next action.

## 4. Problems and Observed Phenomena

Some old state actions were visually weak.

Blocking delays and missing start/end messages made it difficult to know whether a command was still executing.

Tight cables affected head motion.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Command timing and servo execution were unsynchronized.

Without explicit protocol feedback, software/hardware faults looked similar.

Pan/tilt mechanics required cable slack.

Attempts

Add START/DONE/READY messages.

Make state feedback more visible.

Run chained BEHAVIOR_TEST.

Final solution / decision: Established the low-level behavior protocol: while ESP32 executes an action, Python/user waits for READY_FOR_NEXT_COMMAND. Mechanical cable management reduced motion interference.

## 6. Test / Experimental Results

Power/cable engineering completed.

Behavior Firmware v2 ran stably.

Atlas 5.0 gained unified state feedback.

## 7. Remaining Issues

Python still needed to take over the serial protocol from manual Serial Monitor use.

## 8. Next Work Plan

Enter Stage 5-8 Python HAL, state machine, simulated brain, and text-dialogue flow.

# 11. 2026-07-08 - Atlas 5.0 Stage 5-8 - Python HAL, State Machine, Brain Simulation, Dialogue Flow

Recorder: Eric    Phase: Code Debugging / Test Optimization

## 1. Tasks and Completion

Planned tasks

Control ESP32 from Python.

Build a Python behavior state machine and simulated input layer.

Create a text dialogue flow that triggers hardware states.

Actual completed work

Completed Python HAL PING and behavior tests.

Built the behavior-state-machine menu.

Built Brain Simulation mapping text intents to hardware states.

Completed Dialogue Flow so typed input produced replies and physical feedback.

## 2. Tools, Materials, and Software

Software: Python, pyserial, atlas5_hal_ping_test.py, atlas5_hal_behavior_test.py, atlas5_stage6_behavior_state_machine.py, atlas5_stage7_brain_simulation.py, atlas5_stage8_dialogue_flow.py. Hardware: ESP32, pan/tilt, LEDs, 4xAA holder.

## 3. Process Record - Key Operations / Parameters / Steps

Built the HAL first: open serial, send commands, wait for READY.

Validated PING/LEDTEST/ARM before higher layers.

Added a simple intent mapper and dialogue log only after HAL and state machine worked.

## 4. Problems and Observed Phenomena

Python could not open the port while Serial Monitor was active.

Sending commands without waiting for READY caused missed behavior.

Intent recognition was rule-based rather than an LLM.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Only one process can own a serial port.

ESP32 actions need execution time.

The engineering goal was a stable closed loop, not open-domain conversation.

Attempts

Close Serial Monitor before Python.

Honor READY_FOR_NEXT_COMMAND.

Validate text input before microphone integration.

Final solution / decision: Created a layered system: HAL -> State Machine -> Brain Simulation -> Dialogue Flow. Each layer was validated before integration.

## 6. Test / Experimental Results

Python reliably controlled ESP32.

Text input -> intent -> behavior state -> hardware feedback worked.

The system was ready for real voice input.

## 7. Remaining Issues

Real microphone input was not integrated yet.

Voice output and camera were still separate.

## 8. Next Work Plan

Enter Stage 9-10 real voice-input preparation and ASR selection.

# 12. 2026-07-09 - Atlas 5.0 Stage 9-10 - Real Voice Input Preparation, Vosk Failure, and Route Adjustment

Recorder: Eric    Phase: Code Debugging / Test Optimization

## 1. Tasks and Completion

Planned tasks

Add microphone recording and LISTENING state.

Try Vosk offline speech recognition.

Fall back to the proven Atlas 4.0 voice stack if Vosk is unstable.

Actual completed work

Completed Stage 9 microphone recording and WAV saving with manual confirmed text.

Tried Vosk but hit MODEL_PATH and model-creation errors.

Confirmed Python 3.12 direction but model-path/model-structure problems remained.

Decided to abandon Vosk for this version and reuse sounddevice + SpeechRecognition + pyttsx3.

## 2. Tools, Materials, and Software

Software: Python, sounddevice, scipy, SpeechRecognition, pyttsx3, Vosk (attempted then abandoned), PyCharm. Hardware: PC microphone/speaker, ESP32, pan/tilt, LEDs.

## 3. Process Record - Key Operations / Parameters / Steps

First completed a recording layer without automatic recognition.

Tried Vosk with expected model subdirectories.

Traced NameError and model-load failures.

Compared the failing route with previously successful Atlas 4.0 voice code.

## 4. Problems and Observed Phenomena

NameError: MODEL_PATH is not defined.

After fixing definition order, Vosk still failed to create a model.

Local model path/structure compatibility remained a high-risk dependency.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

MODEL_PATH was referenced too early.

Vosk failure could involve path, structure, model integrity, or environment compatibility.

Offline ASR added complexity unrelated to the immediate project goal.

Attempts

Check Python version and model path.

Attempt a path fix.

Reuse the already-proven online SpeechRecognition route.

Final solution / decision: Redefined Stage 10 as Stage 10B using sounddevice recording, Google SpeechRecognition, and pyttsx3 instead of Vosk.

## 6. Test / Experimental Results

Stage 9 recording flow completed.

The recognition architecture was simplified to a known-good approach.

The route was ready for the next day's full voice loop.

## 7. Remaining Issues

Stage 10B integration remained.

Google SpeechRecognition required network access.

## 8. Next Work Plan

Build the real voice input + voice output + ESP32 behavior loop.

# 13. 2026-07-10 (Morning) - Atlas 5.0 Stage 10B - Google Voice + TTS + ESP32 Behavior Loop

Recorder: Eric    Phase: Code Debugging / Test Optimization

## 1. Tasks and Completion

Planned tasks

Reuse the proven Atlas 4.0 voice approach inside Atlas 5.0.

Implement speech -> recognition -> reply -> TTS -> ESP32 behavior.

Test English and Chinese input.

Actual completed work

Ran Stage 10B using SpeechRecognition and pyttsx3.

Successfully recognized English and Chinese examples.

Logged several intent classes including project questions, self-introduction, encouragement, normal questions, and greeting.

Triggered SUCCESS and ENCOURAGE hardware states from recognized speech.

## 2. Tools, Materials, and Software

Software: Python, sounddevice, scipy, numpy, SpeechRecognition, pyttsx3, pyserial, atlas5_stage10b_google_voice_and_tts.py. Hardware: PC microphone/speaker, ESP32, pan/tilt, LEDs, 4xAA holder.

## 3. Process Record - Key Operations / Parameters / Steps

Performed PING, LEDTEST, ARM, and IDLE before voice interaction.

Used LISTENING during recording and THINKING during recognition.

Mapped intents to behavior states and speech responses.

## 4. Problems and Observed Phenomena

At one point speech worked while hardware did not respond.

ESP32 body firmware may have been overwritten by another test sketch.

Serial Monitor could still block Python access.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

ESP32 only runs the most recently flashed firmware.

If body firmware is replaced, Python state commands are no longer recognized.

Serial ownership conflicts remain possible.

Attempts

Validate voice separately.

Check power, wiring, and serial ownership.

Reflash the stable ESP32 body firmware.

Final solution / decision: Restored and preserved stable body firmware. Python became responsible for sending state commands; ESP32 remained responsible for LED and servo behavior.

## 6. Test / Experimental Results

All Stage 10B tests passed.

Atlas 5.0 achieved a real voice input + voice output + hardware behavior loop.

Successful dialogue records were generated.

## 7. Remaining Issues

Local memory, OLED state display, and camera input still needed integration.

## 8. Next Work Plan

Enter Stage 11 memory and Stage 12 OLED/camera integration.

# 14. 2026-07-10 (Afternoon) - Atlas 5.0 Stage 11-12 - Local Memory, OLED, Camera, and Safe Mode

Recorder: Eric    Phase: Hardware Integration / Code Debugging / Test Optimization

## 1. Tasks and Completion

Planned tasks

Add local JSON memory.

Integrate OLED state display.

Integrate the EMEET C950 through PC/OpenCV.

Fix servo twitching and camera-window input issues.

Actual completed work

Built a local memory record with student_name, dialogue_count, last_user_text, last_intent, last_emotion, last_topic, and last_reply.

Connected OLED and flashed OLED-capable body firmware.

Diagnosed the head pointing downward after ARM as a mechanical servo-neutral problem.

Opened the C950 successfully through OpenCV on the PC.

Built a terminal-control Safe Mode version for camera + hardware behavior.

## 2. Tools, Materials, and Software

Software: Python, OpenCV, pyserial, Arduino IDE, Adafruit SSD1306/GFX, ESP32Servo, atlas5_stage12c_camera_terminal_control_safe.py. Hardware: ESP32, OLED, C950, pan/tilt, LEDs, 4xAA holder, PC.

## 3. Process Record - Key Operations / Parameters / Steps

Tested OLED first without camera.

Tested body states after enabling servo power.

Re-centered the tilt servo mechanically around the software neutral.

Tested camera independently before camera + ESP32 integration.

Changed behavior to ARM -> ACTION -> DISARM to reduce continuous servo holding.

Moved command input from OpenCV-window keys to the PyCharm terminal.

## 4. Problems and Observed Phenomena

OLED/head pointed downward after ARM.

Servos twitched continuously during camera-control tests.

Initial actions worked but later terminal input sometimes seemed ignored.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Mechanical tilt neutral did not match software 90 degrees.

Small servos were being held energized under OLED load.

The old camera-window version only listened for keys when the OpenCV window had focus.

Attempts

Reinstall the horn at software neutral or calibrate the front angle.

Use Safe Mode with short ARM windows.

Use terminal input instead of OpenCV-window key focus.

Final solution / decision: Adopted OLED body firmware + PC-hosted C950 + Python terminal-control Safe Mode. This preserved vision preview while reducing servo stress and input-focus failures.

## 6. Test / Experimental Results

OLED + Python control passed.

C950 opened successfully.

Terminal commands triggered hardware states.

Atlas gained a vision-input foundation + OLED status + safer physical behavior.

## 7. Remaining Issues

Automatic visual recognition was not yet the final control path.

Servos should not remain ARM-held for long periods.

## 8. Next Work Plan

Integrate the final voice, camera, OLED, LED, pan/tilt, safe actions, and logging program.

# 15. 2026-07-10 (Evening) - Atlas 5.0 Final Stable Program v1 + Atlas 1.0-5.0 Archive

Recorder: Eric    Phase: Code Integration / Test Optimization / Reporting

## 1. Tasks and Completion

Planned tasks

Integrate camera and voice into the final Atlas 5.0 program.

Integrate Atlas 1.0-5.0 capabilities into one master program.

Merge JSON and TXT records.

Create the complete engineering-log archive.

Actual completed work

Completed Atlas 5.0 Final Stable Program v1 with camera preview, terminal menu, real audio recording, SpeechRecognition, pyttsx3, ESP32 OLED/LED/pan-tilt control, and Safe Mode.

Tested English and Chinese dialogue with all major physical modules running together.

Saved camera snapshots.

Integrated the all-version main program.

Merged Stage 10B and Final Stable JSON records into 11 total runtime records.

Combined TXT records and prepared the engineering-log document.

## 2. Tools, Materials, and Software

Software: Python, OpenCV, sounddevice, SpeechRecognition, pyttsx3, pyserial, JSON, TXT, Word/docx, atlas_1_2_3_4_5_full_main.py, atlas_all_versions_final_data.json. Hardware: ESP32, OLED, C950, PC microphone/speaker, pan/tilt, LEDs, 4xAA holder.

## 3. Process Record - Key Operations / Parameters / Steps

Ran PING/LEDTEST/OLEDTEST before enabling the servo battery.

Used ARM -> IDLE -> DISARM as the initial physical check.

Kept the camera window open but used terminal commands.

Saved recognized text, intent, reply, state, and snapshot paths.

Integrated 1.0-4.0 and 5.0 through an outer master menu.

## 4. Problems and Observed Phenomena

Multi-version code and data were scattered.

The final multimodal program had many peripheral dependencies.

Google SpeechRecognition still required Internet access.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Five project versions naturally produced many files and schemas.

Any occupied/unplugged peripheral could fail in a multimodal demo.

ASR was cloud-backed rather than offline.

Attempts

First stabilize the 5.0 final program independently.

Merge JSON, then TXT, then the engineering log.

Keep Safe Mode to protect servos.

Final solution / decision: Locked the Atlas 5.0 stable demonstration architecture: ESP32 body firmware remains stable; Python owns voice/camera/menu/logging/data integration; physical actions are triggered by state commands and followed by DISARM.

## 6. Test / Experimental Results

Atlas 5.0 final integration passed.

Atlas 1.0-5.0 master program existed.

JSON/TXT records were archived.

The project now had a strong evidence base for code, data, engineering logs, and video.

## 7. Remaining Issues

Automatic face/motion detection remained future work.

ROS2 remained a major next-system upgrade.

Final demo filming and portfolio packaging remained.

## 8. Next Work Plan

Film the Atlas 5.0 final demo.

Protect the 5.0 archive as read-only.

Begin Atlas 6.0 as a new development branch rather than editing the stable 5.0 build.

# 16. 2026-07-13 to 2026-07-19 - Atlas 6.0 Stage 0-2 - Freeze Atlas 5.0, Install Robotics Environment, Build Engineering Skeleton

Recorder: Eric    Phase: Archive Protection / Environment Setup / System Architecture

## 1. Tasks and Completion

Planned tasks

Preserve Atlas 5.0 as a stable, read-only baseline.

Create a separate Atlas_6.0_Development workspace.

Prepare WSL2, Ubuntu 24.04, ROS2 Jazzy, Arduino/ESP32 tooling, Git, and a documented engineering structure.

Separate high-level robotics planning from ESP32 real-time control.

Actual completed work

Created a protected 5.0 archive and a separate 6.0 development copy.

Confirmed the Windows + WSL2 + Ubuntu 24.04 + ROS2 Jazzy direction.

Established the 6.0 project skeleton with docs, tests, test_data, ESP32 firmware folders, and development rules.

Defined early architecture: ROS2 for high-level organization/safety/logging, ESP32 for GPIO/PWM/encoder/sensor real-time work, and a safety layer with final veto authority.

## 2. Tools, Materials, and Software

Software: Windows, WSL2, Ubuntu 24.04, ROS2 Jazzy, Arduino IDE, ESP32 board support, Git, Python tools, project Markdown documentation. Hardware: existing Atlas 5.0 head preserved; mobile-base hardware not yet powered.

## 3. Process Record - Key Operations / Parameters / Steps

Kept Atlas 5.0 files outside the active 6.0 development path.

Created engineering standards for hardware baseline, GPIO map, safety, communication protocol, acceptance tests, decision log, changelog, and engineering log.

Intentionally prevented early 6.0 scope from expanding into SLAM or a large AI-agent architecture before the mobile base was reliable.

## 4. Problems and Observed Phenomena

Atlas 5.0 was a stable interactive robot but not yet a reliable mobile robotics platform.

ROS2 knowledge and mobile-base engineering had to be added without damaging the working 5.0 system.

Hardware details and GPIO assignments were not yet sufficiently confirmed for powered motor testing.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Modifying the 5.0 archive would destroy the project baseline.

Mixing high-level software, motor power, and unverified GPIO at the same time would create too many variables.

An engineering-documentation layer was needed to keep later decisions reproducible.

Attempts

Freeze 5.0; develop only in a copied 6.0 branch.

Document before wiring.

Use staged acceptance gates and full replacement files rather than ad-hoc code patches.

Final solution / decision: Atlas 6.0 began as a new mobile-robot engineering program with a protected predecessor, reproducible workspace, formal safety rules, and an explicit real-time / high-level architecture split.

## 6. Test / Experimental Results

5.0 remained protected.

ROS2/Ubuntu/Git/Arduino development environment was prepared.

6.0 engineering skeleton and documentation standards were established.

## 7. Remaining Issues

Mobile chassis was not yet powered.

Final motor driver, encoder configuration, and power architecture still required real testing.

## 8. Next Work Plan

Proceed to chassis assembly and software-only ROS2 fundamentals while keeping the motor system unpowered.

# 17. 2026-07-22 - Atlas 6.0 Stage 3 + Parallel ROS2 Fundamentals - Mechanical Chassis and Software Pre-Learning

Recorder: Eric    Phase: Mechanical Assembly / ROS2 Learning / Software-Only Testing

## 1. Tasks and Completion

Planned tasks

Complete the empty mobile chassis without powering the motors.

Measure the physical robot geometry and verify free mechanical movement.

Use the waiting time to learn ROS2 nodes, topics, publishers/subscribers, launch files, and graph inspection without controlling the robot.

Actual completed work

Assembled the mobile base and manually checked forward/backward/turning motion.

Measured wheel/chassis geometry for later control work.

Identified a small acrylic-chassis crack and treated it as a structural item to monitor before powered tests.

Loaded the ROS2 Jazzy environment, used the Atlas workspace, and validated simple command_node -> display_node communication as a software-only exercise.

## 2. Tools, Materials, and Software

Software: Ubuntu 24.04, ROS2 Jazzy, colcon workspace, rosdep, Git, Python ROS2 nodes. Hardware: unpowered chassis, wheels, caster, motors, acrylic plates.

## 3. Process Record - Key Operations / Parameters / Steps

Kept the mobile base unpowered while checking wheel alignment, friction, caster height, and structure.

Ran ROS2 communication exercises separately from hardware so a ROS learning mistake could not command the motors.

Used Git and workspace structure as part of the engineering workflow rather than treating code files as isolated sketches.

## 4. Problems and Observed Phenomena

The chassis had an acrylic crack that needed monitoring.

An initially available 7.5 A automotive fuse was recognized as inappropriate for the intended protection target.

ROS2 exercises were ahead of the physical mainline and therefore could not be treated as real robot integration.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Mechanical damage must be separated from electrical/software debugging.

A protection device is only useful when its rating matches the expected current and wiring.

Software simulation and conceptual learning are not proof of powered hardware behavior.

Attempts

Temporarily stabilize and monitor the acrylic area; do not use it as proof of structural readiness by itself.

Switch the intended protection direction to an appropriately rated 2 A fuse/holder.

Label ROS2 work as parallel preparation, not as completion of the hardware stages.

Final solution / decision: Completed the unpowered chassis and a software-only ROS2 learning block while preserving the project rule that real motor stages require separate physical acceptance.

## 6. Test / Experimental Results

Chassis could be manually moved without obvious binding.

ROS2 publisher/subscriber fundamentals worked.

The power-protection plan was corrected before powered testing.

## 7. Remaining Issues

Powered motor behavior remained unverified.

The acrylic crack required continued observation.

Final motor driver/power wiring still needed confirmation.

## 8. Next Work Plan

Prepare safe powered motor testing with current protection, physical disconnect capability, and staged measurements.

# 18. 2026-07-23 to 2026-07-27 - Atlas 6.0 Stage 4 Preparation - Power, Protection, Motor/Encoder Identification

Recorder: Eric    Phase: Electrical Safety Preparation / Hardware Identification

## 1. Tasks and Completion

Planned tasks

Prepare the mobile base for the first powered test without yet driving the robot on the ground.

Confirm motor wiring, encoder wire groups, driver pins, power isolation, and emergency-stop strategy.

Keep encoder signals isolated until basic motor direction is proven.

Actual completed work

Prepared a 2 A fuse direction, fuse holder, initial approximately 6 V motor-power concept, and a self-locking emergency-stop idea.

Inspected driver and motor wiring.

Confirmed that the Hiwonder six-wire motor assembly contains two motor-power wires plus four encoder-related wires.

Kept the first motor test scoped to motor power/control only.

## 2. Tools, Materials, and Software

Hardware: ESP32 DEVKIT_C, Hiwonder geared encoder motors, motor-driver hardware under evaluation, fuse/fuse holder, emergency-stop hardware, battery/power sources, jumper and power wires. Software: Arduino IDE and staged motor-test firmware.

## 3. Process Record - Key Operations / Parameters / Steps

Separated motor power wires from encoder wires.

Kept wheels off the ground for first power tests.

Defined stop conditions for sparks, odor, heat, stalled buzzing, uncontrolled motion, or uncertain polarity.

## 4. Problems and Observed Phenomena

The project had multiple possible power/protection approaches.

Encoder wires made the motor harness more complex than a simple two-wire motor.

Using an incorrectly rated protection device could create a false sense of safety.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Motor control, encoder measurement, and high-level navigation should not be introduced in the same first-power experiment.

Protection must be in the real high-current path, not substituted by a regulator module.

Common ground and power-domain separation had to be explicit.

Attempts

Reduce the first powered test to one motor channel at a time.

Keep encoder wires disconnected until motor polarity/direction is stable.

Use clear stop conditions and record real observations instead of assuming success from compiled code.

Final solution / decision: Established a controlled pre-power checklist and separated the motor-power problem from later encoder and ROS2 work.

## 6. Test / Experimental Results

Hardware roles were clearer.

Unsafe shortcuts such as treating a regulator as a fuse were rejected.

Project was ready for a tightly bounded Stage 4 motor bench test.

## 7. Remaining Issues

Powered motor behavior still required a real Stage 4 bench test.

Encoder integration and ROS2 control remained intentionally deferred.

## 8. Next Work Plan

Run the first real driver/motor test and stop immediately if the physical system does not behave as expected.

# 19. 2026-07-28 - Atlas 6.0 Stage 4 - First Powered Motor Test Failure and Driver Redesign

Recorder: Eric    Phase: Real Hardware Test / Failure Analysis / Architecture Change

## 1. Tasks and Completion

Planned tasks

Validate ESP32 -> motor driver -> left/right motor control with wheels suspended.

Confirm that software commands and serial communication translate into real motor movement.

Collect physical evidence instead of declaring the stage successful from code output alone.

Actual completed work

ESP32 firmware and serial command handling operated, but the expected motor movement did not occur in the initial driver path.

Commands such as low and increased PWM were tested while the physical motor remained nonfunctional.

Without a multimeter available in the moment, the system could not reliably separate power-path, ground, driver, or current-delivery causes.

The Stage 4 architecture was redesigned around an L298N path with staged voltage/current measurement.

## 2. Tools, Materials, and Software

Software: dedicated safe motor-test firmware and Serial Monitor. Hardware: ESP32 DEVKIT_C, original driver test path, Hiwonder motors, power/protection hardware; L298N selected for the redesigned path.

## 3. Process Record - Key Operations / Parameters / Steps

Observed the critical engineering distinction: firmware/serial success did not equal electromechanical success.

Did not fabricate a motor-pass result.

Redesigned the test plan to require measurement and single-motor validation before dual-motor operation.

## 4. Problems and Observed Phenomena

The program accepted commands but the motor did not move.

Cause could not be isolated without measuring the actual electrical nodes.

Continuing to change code would not answer whether the fault was electrical.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Possible causes included power delivery, missing common ground, driver behavior, connection errors, insufficient current, or motor-driver incompatibility.

A multimeter and staged isolation were more valuable than additional software changes.

Attempts

Stop the failed driver path rather than repeatedly increasing PWM.

Adopt L298N for a new Stage 4-R bench path.

Lock a clear GPIO map and validate one motor, then the other, then both.

Final solution / decision: The failed first-power test became a documented architecture pivot. Atlas 6.0 moved to the L298N-based motor-control chain later used by the integrated release.

## 6. Test / Experimental Results

Software and serial interface were confirmed independently of motor movement.

The failure was preserved as project evidence.

A higher-success Stage 4-R test architecture was established.

## 7. Remaining Issues

L298N had not yet been fully accepted at the beginning of this record.

Encoder integration still remained separate.

## 8. Next Work Plan

Complete Stage 4-R with the L298N, lock motor GPIO, and validate physical direction/stop behavior before adding encoder feedback.

# 20. 2026-08-07 - Atlas 6.0 Stage 4-R / Stage 5 - L298N Drive Recovery, Direction Mapping, and Basic Movement

Recorder: Eric    Phase: Motor Bring-Up / Direction Debugging / Safety Validation

## 1. Tasks and Completion

Planned tasks

Establish a reliable L298N drive path and lock the final motor GPIO.

Confirm left/right motor direction and STOP behavior before closed-loop work.

Keep hardware wiring stable and correct direction errors in software where practical.

Actual completed work

Locked the L298N motor-control map: left ENA13/IN1=14/IN2=4; right ENB33/IN3=32/IN4=23.

A real TESTL 100 observation showed that the physical left wheel moved backward when commanded forward.

Rather than rewiring the now-stable harness, the left motor direction was inverted in software while the right motor kept its physical polarity.

The mobile-base control path progressed from non-movement into repeatable real motor operation.

## 2. Tools, Materials, and Software

Software: Stage 4-R motor firmware, early Stage 6/7 firmware family. Hardware: ESP32 DEVKIT_C, L298N, two Hiwonder geared encoder motors, suspended chassis, controlled motor-power source.

## 3. Process Record - Key Operations / Parameters / Steps

Tested one physical motor at a time before dual-motor commands.

Used software inversion to preserve wiring stability.

Kept STOP/DISARM and bounded-duration movement as safety requirements.

## 4. Problems and Observed Phenomena

One physical motor direction did not match the logical command.

Low-PWM starts could buzz or fail to overcome static friction in later integrated tests.

Changing physical wiring after encoder work began would create mapping risk.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Motor polarity and encoder polarity are separate engineering problems.

DC motors may need extra starting torque even if a lower PWM is sufficient once already moving.

A stable harness is valuable; direction can be normalized in software.

Attempts

Invert the left motor in software.

Preserve the right motor mapping.

Later add a short startup-boost pulse rather than permanently raising cruise PWM.

Final solution / decision: Established the motor-direction abstraction that allowed higher layers to use logical FORWARD/BACK/LEFT/RIGHT without rewiring the physical drivetrain.

## 6. Test / Experimental Results

L298N became the working motor-driver architecture.

Basic motor direction and stopping behavior were reproducible.

The drivetrain was ready for encoder calibration.

## 7. Remaining Issues

Encoder channel mapping, sign, PPR, and physical distance calibration were still required.

Closed-loop straight driving had not yet been tuned.

## 8. Next Work Plan

Proceed to Stage 6 encoder mapping, sign learning, pulses-per-revolution measurement, and distance calibration.

# 21. 2026-08-08 - Atlas 6.0 Stage 6 - Dual Encoder Mapping and Calibration

Recorder: Eric    Phase: Encoder Debugging / Calibration / Distance Control

## 1. Tasks and Completion

Planned tasks

Identify which GPIO pair belongs to each physical wheel.

Ensure forward encoder sign is correct rather than hiding errors with abs().

Measure real wheel travel and pulses per revolution.

Enable pulse- and distance-based stopping with independent wheel completion.

Actual completed work

Locked physical right encoder to GPIO18/19 and physical left encoder to GPIO25/26.

Added explicit forward-sign learning/check logic so the program verifies encoder direction after reboot.

Final measured PPR values were LEFT=632.05 and RIGHT=634.17.

Corrected the wheel calibration to a measured rolling circumference of 21.40 cm.

Added target-pulse and target-distance commands, independent left/right stopping, stall timeout, total timeout, and spike protection.

## 2. Tools, Materials, and Software

Software: Atlas 6.0 Stage 6C V2/V3/V4 family. Hardware: ESP32 DEVKIT_C, L298N, dual Hiwonder encoder motors, suspended chassis, tape/ground reference for rolling circumference.

## 3. Process Record - Key Operations / Parameters / Steps

Separated raw GPIO-group names from physical left/right meaning.

Required short encoder-direction checks before allowing target-distance movement.

Replaced an earlier incorrect wheel-travel assumption with the directly measured 21.40 cm rolling circumference.

Used per-wheel PPR rather than a single shared nominal value.

## 4. Problems and Observed Phenomena

Early encoder data could appear unstable or have unexpected sign.

Raw channel names did not initially map cleanly to physical left/right labels.

An early wheel-circumference value was not physically correct for final distance control.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Encoder sign errors must be fixed at the mapping layer, not hidden by absolute-value calculations.

Left and right encoders had slightly different PPR and therefore required independent conversion.

Real rolling circumference is more useful than relying on nominal wheel geometry.

Attempts

Lock physical-to-GPIO mapping.

Use forward-sign learning/check commands.

Measure multiple revolutions and preserve independent PPR constants.

Update distance conversion to the measured 21.40 cm rolling circumference.

Final solution / decision: Atlas 6.0 gained a trustworthy encoder foundation for later closed-loop motion: physical wheel identity, direction, PPR, and travel conversion were all explicit and testable.

## 6. Test / Experimental Results

LEFT_PPR = 632.05.

RIGHT_PPR = 634.17.

Measured rolling circumference = 21.40 cm.

Target-distance logic included 700 ms stall protection, bounded total runtime, and abnormal-pulse checks.

## 7. Remaining Issues

Same-PWM open-loop driving still produced left/right progress mismatch.

Distance stopping still needed better synchronization and endpoint behavior.

## 8. Next Work Plan

Enter Stage 7 closed-loop straight synchronization and precision distance stopping.

# 22. 2026-08-12 and subsequent Stage 7 iterations - Atlas 6.0 Stage 7A-7D - Closed-Loop Straight Driving and Distance Accuracy

Recorder: Eric    Phase: Control Tuning / Closed-Loop Motion / Iterative Test Optimization

## 1. Tasks and Completion

Planned tasks

Quantify left/right mismatch under the same PWM.

Add conservative synchronization without destabilizing already reliable low-speed operation.

Build precise distance stopping while preserving STOP, stall, reverse-pulse, and timeout safety.

Iterate based on real test data instead of theoretical motor symmetry.

Actual completed work

Open-loop test f 80 1000 was repeated five times and remained stable.

Average raw progress was approximately 928.6 pulses left vs 916.6 right; after independent-PPR normalization, the left side led by about 1.64%.

SYNC V1 reduced average absolute left/right progress error to about 1.268% and reduced worst-case error from about 4.20% to 1.59%.

V2 kept symmetric adaptive compensation rather than adding a fixed left/right bias because the leading wheel was not always the same in every run.

FDS distance control evolved through multiple versions: dynamic braking, adaptive pulse approach, recovery cycles, dynamic cycle budgets, and continuous low-PWM control.

Ground testing showed that PWM 80 could sustain movement, while very short stop/start pulses could fail to overcome static friction reliably; the later continuous-distance design therefore kept the motor at the lowest reliable continuous PWM and adjusted the lagging wheel modestly.

## 2. Tools, Materials, and Software

Software: Stage 7 Closed-Loop Straight V1/V2; Stage 7B Closed-Loop Distance V1-V5; Stage 7D Continuous Distance V6. Hardware: Atlas mobile base with L298N and dual encoders.

## 3. Process Record - Key Operations / Parameters / Steps

Used normalized progress rather than raw pulse difference because the two encoders have different PPR.

Updated compensation every 50 ms and bounded the extra PWM added to the lagging wheel.

Introduced L298N dynamic braking for endpoint control.

When long continuous motion overshot short targets, changed to pulse-approach control; when very short pulses became unreliable on the ground, moved again to continuous low-speed FDS with independent early braking.

Adjusted cycle limits after real fds 15 tests required 86, 87, and 89 cycles, proving that a fixed budget could be too small for longer targets.

## 4. Problems and Observed Phenomena

Open-loop left/right mismatch was real but variable.

The first synchronized controller improved error but did not meet an arbitrary 30% improvement target.

Continuous PWM caused significant overshoot for short targets.

Pulse-by-pulse stopping improved endpoint control but could fail to restart the motors from rest on the ground.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Fixed bias was risky because the identity of the leading wheel could change between trials.

Motor inertia and L298N braking behavior mattered near the endpoint.

Static friction made extremely short low-PWM bursts unreliable even when the same PWM worked in continuous motion.

Attempts

Use bounded adaptive compensation rather than permanent asymmetric PWM.

Use braking and per-wheel completion.

Base final control on measured motor startup/continuous behavior.

Keep formal pass criteria separate from whether the safety logic completed successfully.

Final solution / decision: Established a data-driven closed-loop motion stack that was later carried into wireless and sensing stages without re-tuning the drivetrain every time.

## 6. Test / Experimental Results

Open-loop five-run baseline recorded.

SYNC V1 average absolute error approximately 1.268%; worst-case reduced to 1.59%.

FDS pass criteria were explicit: final distance error <=10% per wheel and left/right final mismatch <=5%.

Distance control retained stall, reverse-pulse, abnormal-pulse, STOP, and total-time protections.

## 7. Remaining Issues

Long-range navigation was not the goal of 6.0; the controller was optimized for reliable tabletop/mobile-base experiments.

Wireless communication still had to replace the dragging data cable.

## 8. Next Work Plan

Add a Wi-Fi/TCP bridge without changing the proven motor/encoder/FDS behavior.

# 23. Mid-August 2026 - Atlas 6.0 Stage 7C - Wireless Bridge and Removal of the Driving Data Cable

Recorder: Eric    Phase: Communication Integration / Safety Regression

## 1. Tasks and Completion

Planned tasks

Remove the physical data cable from normal movement so cable drag no longer affects driving.

Add Wi-Fi/TCP as a second command path while keeping the same processCommand() logic.

Preserve Stage 7B drivetrain parameters and require heartbeat-based fail-safe stopping.

Actual completed work

Created a Wi-Fi access point on the ESP32 and a TCP command server.

USB Serial and Wi-Fi TCP were routed into the same command parser so wireless control did not fork the motor logic.

Runtime reports were mirrored to USB serial and the active TCP client.

Added a 350 ms TCP heartbeat timeout that forces STOP and DISARM when network control disappears.

This architecture later evolved into the final ATLAS_6_0 AP / TCP control path used by the integrated release.

## 2. Tools, Materials, and Software

Software: Atlas_6_0_Stage_7C_Wireless_Bridge_V1.ino and later integrated firmware. Hardware: ESP32 Wi-Fi, L298N, dual encoders, mobile base, PC wireless connection.

## 3. Process Record - Key Operations / Parameters / Steps

Explicitly froze drivetrain tuning before adding the network layer.

Used one command parser for both USB and TCP.

Added heartbeat safety before using wireless motion commands.

Tested communication as its own layer rather than changing motor tuning at the same time.

## 4. Problems and Observed Phenomena

A tethered USB/data cable physically affected free movement.

A wireless command path creates a new failure mode: client disconnect or heartbeat loss.

Network debugging can become confused with motor-control debugging if both are modified at once.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Wireless control is only acceptable when loss of communication becomes a stop condition.

Duplicated command parsers could drift out of sync.

The safest architecture is one low-level command/safety implementation with multiple transport inputs.

Attempts

Keep motor/encoder code unchanged.

Add TCP only at the transport boundary.

Mirror status output and implement independent heartbeat timeout.

Final solution / decision: Atlas 6.0 became untethered at the command layer while retaining a deterministic ESP32 safety boundary. Communication loss was treated as a safety event, not a reason to keep moving on the last command.

## 6. Test / Experimental Results

Wi-Fi/TCP command path worked.

350 ms heartbeat fail-safe was established and later retained in the final integrated firmware.

Cable drag was removed from normal motion tests.

## 7. Remaining Issues

The first Stage 7C AP credentials/port were development values and changed later in the integrated release.

Distance sensing and autonomous obstacle behavior still had to be integrated.

## 8. Next Work Plan

Integrate US-100 distance sensing as a physical veto layer before adding camera intelligence.

# 24. Mid-to-Late August 2026 - Atlas 6.0 Stage 8A-8B - US-100 Sensor Safety Gate and Suspended Integration

Recorder: Eric    Phase: Distance Sensing / Safety-State Integration / Real Motor Validation

## 1. Tasks and Completion

Planned tasks

Add the US-100 without weakening the already-proven encoder/motor safety logic.

Make invalid or dangerously close distance data block forward motion.

Validate sensor logic with motor output disabled before enabling real motor output.

Require re-ARM after a safety event instead of automatically resuming motion.

Actual completed work

Connected US-100 in Trigger/Echo mode using GPIO27 (Trig) and GPIO34 (Echo).

Built Stage 8A with MOTOR_OUTPUTS_ENABLED=false so the entire safety state machine could be exercised without motor power.

Used FORCE_STOP, CAUTION, CLEAR, STARTUP_STOP, and SENSOR_INVALID_STOP states with hysteresis.

For compact no-motor tests, a temporary 25/40/45 cm threshold set was used; the formal Stage 8B real-motor logic restored the 25/50/55 cm policy.

Created Stage 8B suspended integration with real motor output enabled only while wheels remained off the ground and power was controlled.

## 2. Tools, Materials, and Software

Software: Stage 8A Sensor Safety V1/V2 Compact; Stage 8B Suspended Integration V1. Hardware: ESP32, L298N, dual encoders, US-100, mobile base.

## 3. Process Record - Key Operations / Parameters / Steps

First validated sensor states without allowing any motor output.

Required several valid readings before treating the sensor as trustworthy.

Applied hysteresis so the state did not oscillate around a threshold.

After a STOP-class event, required a new ARM rather than silently continuing.

## 4. Problems and Observed Phenomena

Distance thresholds suitable for a large test area were inconvenient for very small indoor bench spaces.

An invalid sensor reading must not be interpreted as safe.

Turning real motors on before logic validation would make sensor bugs physically dangerous.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Safety logic must fail closed.

Hysteresis is needed near distance boundaries.

Software-only and real-output variants should be separate acceptance steps.

Attempts

Use a compact threshold set only for no-motor tests and explicitly restore formal thresholds for powered tests.

Keep invalid data in a stop state.

Enable real motors only after the no-output version passes.

Final solution / decision: Established US-100 as a physical veto authority: obstacle or invalid data can stop motion regardless of higher-level intent.

## 6. Test / Experimental Results

US-100 became part of the low-level safety chain.

Sensor safety could be validated independently of motor actuation.

Suspended motor integration preserved the re-ARM requirement after unsafe states.

## 7. Remaining Issues

Forward-only distance sensing was insufficient for rich left/right route selection.

A scanning mechanism and camera-based perception were still needed for the planned autonomous behavior.

## 8. Next Work Plan

Add servo-based scanning and an autonomous stop/backoff/scan/turn recovery state machine.

# 25. Late August 2026 - Atlas 6.0 Autonomous Obstacle Avoidance - Servo Scan, Startup Boost, and Recovery State Machine

Recorder: Eric    Phase: Autonomous Behavior / Sensor Fusion Preparation / Motion Robustness

## 1. Tasks and Completion

Planned tasks

Use US-100 scanning to select a safer direction after encountering an obstacle.

Make low-speed starts reliable without permanently increasing cruise PWM.

Create a bounded recovery sequence instead of a single open-loop turn command.

Keep no-rear-sensor limitations explicit.

Actual completed work

Added a scan servo and later standardized the continuous sweep sequence LEFT(5) -> CENTER(90) -> RIGHT(175) -> CENTER(90).

Added a short startup boost (later integrated as PWM 220 for 150 ms) to overcome static friction before returning to normal cruise PWM.

Implemented recovery behavior that stops first, performs a short bounded backoff, waits for safe sensor data, scans, then turns toward a safer direction.

Developed integrated candidates through V3.x, including direction-map fixes and auto-recovery logic.

## 2. Tools, Materials, and Software

Software: integrated RC V3.x family including STARTUP_BOOST, DIRECTION_FIXED, and AUTO_RECOVERY variants. Hardware: ESP32, L298N, dual encoders, US-100, scan servo, mobile base.

## 3. Process Record - Key Operations / Parameters / Steps

Kept startup boost short so it solved stall/static-friction behavior without turning into a new high-speed cruise setting.

Made recovery stateful: STOP -> bounded reverse -> scan -> choose turn -> continue only when safe.

Kept rear motion deliberately short because the robot had no rear obstacle sensor.

## 4. Problems and Observed Phenomena

At low PWM the motors could buzz or fail to start from rest.

Obstacle detection alone did not tell the robot which direction to choose.

A recovery routine could become unsafe if reverse duration were unbounded.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Static friction requires more torque at startup than during continuous motion.

US-100 can provide strong physical range evidence but only along the direction it is pointed.

Without a rear sensor, reverse must remain tightly bounded.

Attempts

Use a brief startup boost.

Use servo scanning to collect directional range data.

Use explicit recovery states and stop conditions instead of long blocking movement.

Final solution / decision: Atlas 6.0 moved from command-driven mobility to a real sensor-feedback behavior loop: detect -> stop -> recover -> scan -> turn -> continue.

## 6. Test / Experimental Results

Low-speed startup became more reliable.

Servo-based directional scanning was integrated.

Auto recovery and US-100 veto behavior were incorporated into the integrated firmware family.

## 7. Remaining Issues

Camera perception was still not reliable enough to become a safety authority.

The initial image-scanning idea required its own dataset and validation process.

## 8. Next Work Plan

Integrate the C950 as a PC-side perception sensor, first as a non-controlling baseline, then train models only if the heuristic scan is not reliable.

# 26. Late August 2026 - Atlas 6.0 Stage 12A - C950 Vision Baseline Failure, Dataset Collection, and AI Model Training

Recorder: Eric    Phase: Computer Vision / Dataset Engineering / Model Iteration

## 1. Tasks and Completion

Planned tasks

Use the external EMEET C950 to provide visual free-space / edge information without loading the ESP32.

Begin with a non-controlling perception baseline.

If hand-designed image thresholds are not reliable across scenes, collect labeled data and train models rather than continuously tuning arbitrary thresholds.

Actual completed work

Built a 1280x720 / 30 FPS OpenCV baseline that estimated LEFT/CENTER/RIGHT connected tabletop/free-space from an adaptive appearance reference.

Kept the first vision version completely disconnected from ESP32 and motor control so perception errors could not move the robot.

Observed that the baseline scanning/data interpretation was not reliable enough across different scenes for final avoidance.

Created labeled dataset collectors and split the problem into two tasks: directional traversability (FREE/BLOCKED) and global near-field edge safety (SAFE/EDGE).

Collected multiple scene images, trained models, found remaining misclassifications, retrained, and repeated validation until release criteria could be met.

## 2. Tools, Materials, and Software

Software: Python, OpenCV, Ultralytics YOLO classification, dataset collectors, atlas_vision_stage12a1_camera1.py, edge ROI collector, directional/edge release scripts. Hardware: EMEET C950 connected to the PC; ESP32 and motors deliberately excluded from initial model validation.

## 3. Process Record - Key Operations / Parameters / Steps

Used the lower/near-field part of the camera frame for edge-safety data so the model focused on the physically relevant tabletop boundary.

Kept directional and cliff/edge decisions separate rather than training one overloaded class set.

Saved raw/annotated frames and CSV/validation outputs.

Used validation sets and explicit release scripts instead of accepting a visually impressive single test.

## 4. Problems and Observed Phenomena

Heuristic scanning produced inaccurate or scene-dependent results.

Early trained models still produced analysis errors in some conditions.

Cropping/region selection could remove the very edge evidence needed for safety.

A model can appear good in a demo while still making a dangerous BLOCKED->FREE or EDGE->SAFE error.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Color/appearance heuristics do not generalize reliably across lighting and tabletop conditions.

Safety-critical edge perception needs a different objective from left/right free-space classification.

Validation must prioritize dangerous false-safe errors, not only average accuracy.

Attempts

Collect more representative scenes.

Retrain when direct errors remain.

Use tri-state outputs including UNKNOWN instead of forcing a confident answer.

Require release-freeze checks with zero direct class swaps on the validation set at the selected threshold, while bounding the UNKNOWN ratio.

Final solution / decision: Replaced the unreliable single heuristic scanner with a structured perception pipeline built from data collection, model training, validation, retraining, and release-freeze gates.

## 6. Test / Experimental Results

Created separate directional and edge-safety datasets/models.

Built scripts that refuse to freeze a model when no acceptable threshold exists.

Release logic explicitly treats UNKNOWN as a safe non-advance state rather than a guessed FREE state.

## 7. Remaining Issues

Model validation still needed live integration with a real camera stream.

The final navigation policy still needed to decide how much authority camera vs US-100 should have.

## 8. Next Work Plan

Run live dual-model validation, then define a conservative fusion policy before the camera is allowed to influence navigation.

# 27. Late August 2026 - Atlas 6.0 Vision Release - Dual-Model Validation and Strict Edge-Safety Policy

Recorder: Eric    Phase: Model Validation / Safety Policy / Live Camera Integration

## 1. Tasks and Completion

Planned tasks

Freeze only models that pass validation gates.

Validate the models live on C950 without immediately granting motor authority.

Define a conservative policy that prioritizes cliff/edge safety over directional convenience.

Actual completed work

Created release-freeze tooling for EDGE/SAFE and BLOCKED/FREE models.

Release scripts searched confidence thresholds and rejected a model if no zero-direct-error tri-state threshold was available within the allowed UNKNOWN ratio.

Built a live dual-model pipeline: evaluate the near-field EDGE/SAFE ROI first; only when globally SAFE is established does directional FREE/BLOCKED classification run.

Added temporal smoothing / consecutive-frame confirmation for safe states.

After real testing exposed remaining risk, the final master policy became stricter: a confident EDGE is immediate; weak/not-safe frames revoke SAFE permission; strong SAFE must persist across consecutive frames; LEFT/RIGHT camera authority is disabled for navigation and US-100 becomes the sole physical left/right authority.

## 2. Tools, Materials, and Software

Software: freeze_edge_roi_release.py, resolve_and_freeze_directional_release.py, atlas_live_dual_model_v1.py, atlas_vision_wifi_console_v3_EDGE_STRICT.py, later atlas_6_0_full_master_v1.py. Hardware: C950 on PC, ESP32 over Wi-Fi/TCP.

## 3. Process Record - Key Operations / Parameters / Steps

Treated model freeze as a reproducibility step: selected model file, threshold, class mapping, validation counts, UNKNOWN behavior, and release config were recorded.

Ran camera/model validation separately before full autonomous integration.

Compared SAFE and EDGE diagnostic distributions and refused threshold-only fixes when class separation was insufficient.

Made the final runtime policy more conservative than the raw model output.

## 4. Problems and Observed Phenomena

A model that passes one validation can still be too permissive in live safety conditions.

Directional camera outputs are less physically trustworthy than close-range US-100 measurements for left/right obstacle decisions.

Edge safety needs fast reaction but SAFE permission should be harder to earn.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Classifier confidence is not itself a safety guarantee.

Physical sensors and learned vision have different failure modes.

A safety architecture should use each sensor where it is strongest rather than forcing symmetric authority.

Attempts

Derive a stricter SAFE threshold from measured SAFE and EDGE diagnostics.

Require multiple consecutive strong SAFE frames.

Use camera primarily as strict edge/cliff safety while keeping US-100 as left/right physical authority.

Final solution / decision: Vision became a bounded safety subsystem rather than an unrestricted navigation brain. This was a major architecture correction: learned perception was used where it added value while physical range sensing retained final authority for directional obstacle avoidance.

## 6. Test / Experimental Results

EDGE/SAFE and directional release candidates could be frozen only after explicit validation.

Live dual-model validation pipeline was established.

Final policy implemented fail-closed UNKNOWN behavior and strict SAFE confirmation.

## 7. Remaining Issues

Full-system integration still needed the 5.0 expression/voice layer and the final ESP32 firmware.

Camera processing remained PC-side.

## 8. Next Work Plan

Reintegrate Atlas 5.0 body expression and voice with the 6.0 mobile/safety stack, then freeze the final integrated firmware.

# 28. Late August to Early September 2026 - Atlas 6.0 Final Hardware Integration - V3.5 / V3.5.2 Head Behavior Fix

Recorder: Eric    Phase: Full Hardware Integration / Firmware Stabilization / Safety Regression

## 1. Tasks and Completion

Planned tasks

Combine drivetrain, encoders, US-100 scanning, Wi-Fi/TCP watchdog, OLED, pan/tilt head, and RGB expression on one ESP32 without weakening the movement safety boundary.

Restore the personality/body-expression capability from Atlas 5.0.

Fix head behavior and keep the robot safe on boot, disconnect, and stale perception.

Actual completed work

Integrated the final hardware map: L298N left ENA13/IN1=14/IN2=4; right ENB33/IN3=32/IN4=23; encoders left 25/26 and right 18/19; US-100 27/34; scan servo 21; OLED SDA16/SCL17; pan 5; tilt 22; RGB strip 15.

Finalized boot state as DISARMED / AUTO OFF / US_ONLY / SWEEP ON.

Finalized Wi-Fi AP control in the release line with ATLAS_6_0 and TCP port 8888.

Retained the 350 ms network heartbeat timeout: communication loss forces STOP + DISARM.

Required fresh camera SAFE for FUSION mode and treated stale/UNKNOWN center vision as STOP.

Reintegrated OLED, RGB, and pan/tilt behavior and fixed head-profile behavior in V3.5.2.

Kept pan/tilt electrically quiet until an explicit calibration/state action to reduce unnecessary motion.

## 2. Tools, Materials, and Software

Software: Atlas_6_0_Integrated_RC_V3, V3.2 STARTUP_BOOST, V3.3 DIRECTION_FIXED, V3.4 AUTO_RECOVERY, V3.5 FULL_HARDWARE_INTEGRATION, V3.5.1 RGB fix, V3.5.2 HEAD_BEHAVIOR_FIX. Hardware: complete Atlas 6.0 body.

## 3. Process Record - Key Operations / Parameters / Steps

Integrated one subsystem at a time and kept earlier working versions as rollback points.

Preserved reboot-safe behavior and STOP/DISARM semantics across all integration versions.

Remapped the Atlas 5.0 expression hardware to GPIOs that did not conflict with the 6.0 drivetrain/sensors.

Used explicit build banners/status output to identify which firmware was actually running.

## 4. Problems and Observed Phenomena

Low-speed motor starts required startup boost.

Physical motor direction mapping required a later correction in the integrated release line.

Head motion could interfere with the robot if pan/tilt stayed actively driven without need.

Adding many subsystems to one ESP32 increased the importance of a fixed GPIO map and fail-safe boot.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

The final robot needed a single authoritative hardware map.

Expression features must never bypass motor safety.

Communication and perception failures must transition to STOP/DISARM, not preserve the previous motion state.

Attempts

Keep startup boost bounded.

Lock final direction map in software.

Detach/quiet head outputs when not required.

Preserve explicit safety checks and mirrored status reporting.

Final solution / decision: V3.5.2 became the final Atlas 6.0 ESP32 integrated baseline, combining mobility, sensing, communication safety, and body expression without turning the expression layer into a motor-control dependency.

## 6. Test / Experimental Results

Final firmware booted DISARMED.

Heartbeat timeout and controlled network disconnect both stop/disarm.

US-100 retained physical veto authority.

FUSION required fresh camera-safe state.

OLED/RGB/pan-tilt expression hardware was restored on the mobile robot.

## 7. Remaining Issues

High-level PC-side voice/vision still needed a single master runtime.

Final demo had to prove autonomous behavior, perception, and expression together.

## 8. Next Work Plan

Run the final PC-side master program and demonstrate integrated autonomous behavior with conservative safety.

# 29. Late August to Early September 2026 - Atlas 6.0 Final Master Integration and Demo - Voice, Vision, Expression, Edge Safety, and Autonomous Tabletop Navigation

Recorder: Eric    Phase: System Integration / Demo Validation / Version Closeout

## 1. Tasks and Completion

Planned tasks

Combine C950 vision, ESP32 Wi-Fi/TCP, autonomous obstacle avoidance, strict edge safety, and Atlas 5.0 interaction/personality into one operational master program.

Make human interaction safe even though keyboard input and TTS can block the main loop.

Produce a real end-to-end demo that shows failures, retraining, fixes, and successful autonomous behavior.

Actual completed work

Built atlas_6_0_full_master_v1.py using one shared C950 instance, Wi-Fi/TCP ESP32 control, strict edge-safety calibration, local TTS, and optional English/Chinese voice input.

Re-entered FUSION mode in a DISARMED state; AUTO still required explicit user authorization.

Implemented a safety-first typed-dialogue path that sends AUTO OFF -> STOP -> DISARM before blocking for keyboard/TTS interaction.

Retained the final perception policy: edge model provides strict cliff/edge permission; US-100 is the sole left/right physical authority for navigation.

Completed the development sequence captured in the final demo: define the goal; write US-100 obstacle-avoidance logic; repeatedly tune parameters; achieve ultrasonic-only avoidance; add the C950; discover the initial scan system was inaccurate; collect multi-scene data; train and retrain AI models; validate release criteria; fix residual real-test errors; and finally complete successful integrated testing.

## 2. Tools, Materials, and Software

Software: atlas_6_0_full_master_v1.py, atlas_6_0_brain_integration_v1.py, strict-edge vision console, model-release configs, V3.5.2 ESP32 firmware. Hardware: complete Atlas 6.0 mobile robot, EMEET C950 on the PC side, ESP32 Wi-Fi AP/TCP, L298N, encoders, US-100, scan servo, OLED, pan/tilt, RGB.

## 3. Process Record - Key Operations / Parameters / Steps

Checked release files and edge-safety diagnostic separation before entering live control.

Opened the camera once and shared that stream across the master program instead of competing camera processes.

Connected ESP32 over TCP and kept periodic heartbeat/status traffic.

Kept controller DISARMED until explicit ARM.

Converted voice/text interaction into an explicit safe-stop event before performing blocking user interaction.

## 4. Problems and Observed Phenomena

Initial camera scanning was not accurate enough to trust.

Model training required several iterations and re-validation.

Human interaction could starve heartbeat/control if the robot stayed moving while the program waited for input or TTS.

Combining personality with autonomy risked turning expressive behavior into a safety distraction.

## 5. Root-Cause Analysis and Corrective Actions

Likely causes / engineering interpretation

Perception quality must be proven through data and validation, not appearance in one demo.

Blocking UI/audio work cannot safely coexist with active motion unless motion is stopped first.

Robot personality should be layered above, not below, the safety state machine.

Attempts

Use release-gated models and a strict runtime policy.

Always stop/disarm before blocking interaction.

Keep physical range sensing in the low-level safety loop.

Treat expression/voice as optional higher-level behaviors that cannot bypass STOP/DISARM.

Final solution / decision: Atlas 6.0 closed as a mobile hardware robot that combined real motor control, encoder feedback, closed-loop motion, ultrasonic obstacle sensing, Wi-Fi/TCP safety, PC-side AI vision, autonomous recovery, and the personality/expression lineage from Atlas 5.0.

## 6. Test / Experimental Results

Autonomous obstacle avoidance succeeded after iterative US-100 tuning.

C950 vision progressed from an unreliable baseline to release-gated AI safety models.

Wireless control removed the driving data cable.

Final integrated firmware and master program supported safe edge-aware autonomous tabletop behavior and Atlas-style expression.

The final demo documented both failure/retraining loops and the successful result rather than presenting only the finished system.

## 7. Remaining Issues

Atlas 6.0 did not implement full SLAM/Nav2 mapping; that work was intentionally moved to Atlas 7.0.

Camera compute remained on the PC.

Final mobile power and embedded compute architecture would continue to evolve in 7.0.

## 8. Next Work Plan

Freeze Atlas 6.0 source code, models, logs, and demo evidence.

Use 6.0 as the proven real-time/safety base for Atlas 7.0 ROS2 mapping and navigation.

# Appendix A - Atlas 6.0 Final Technical Baseline

## Final ESP32 / Hardware Map

| Function | Final Atlas 6.0 Mapping |
| --- | --- |
| Left motor L298N | ENA GPIO13; IN1 GPIO14; IN2 GPIO4 |
| Right motor L298N | ENB GPIO33; IN3 GPIO32; IN4 GPIO23 |
| Left encoder | GPIO25 / GPIO26 |
| Right encoder | GPIO18 / GPIO19 |
| US-100 | TRIG GPIO27; ECHO GPIO34 |
| Scan servo | GPIO21 |
| OLED | SDA GPIO16; SCL GPIO17 |
| Head pan | GPIO5 |
| Head tilt | GPIO22 |
| RGB strip | GPIO15 |
| Final Wi-Fi AP | ATLAS_6_0 |
| Final TCP port | 8888 |
| Network fail-safe | 350 ms heartbeat timeout -> STOP + DISARM |

## Key Measured / Tuned Values

Left encoder: 632.05 pulses/revolution.

Right encoder: 634.17 pulses/revolution.

Measured rolling wheel circumference: 21.40 cm (confirmed 2026-08-08).

Stage 7 open-loop baseline at PWM 80 / 1000 ms: five stable trials; left average 928.6 pulses, right average 916.6 pulses; normalized left lead about 1.64%.

Stage 7 SYNC V1: average absolute error about 1.268%; worst error reduced from about 4.20% to 1.59%.

Final integrated startup boost: PWM 220 for 150 ms.

Final integrated scan sequence: LEFT 5 deg -> CENTER 90 deg -> RIGHT 175 deg -> CENTER 90 deg.

Final safety baseline: boot DISARMED; network timeout/disconnect STOP + DISARM; FUSION requires fresh camera SAFE; US-100 has physical veto authority.

# Appendix B - Principal Atlas 6.0 Source / Evidence Files

Atlas_6.0 plan document; Atlas_Master_Context_1.0-6.0.md; docs/09_ENGINEERING_LOG.md.

Atlas 6.0 Stage 4-R engineering plan and operating manual.

Atlas_6_0_Stage_6C_V3/V4.ino.

Atlas_6_0_Stage_7_Closed_Loop_Straight_V1/V2.ino.

Atlas_6_0_Stage_7B_Closed_Loop_Distance_V1-V5.ino.

Atlas_6_0_Stage_7C_Wireless_Bridge_V1.ino.

Atlas_6_0_Stage_7D_Continuous_Distance_V6.ino.

Atlas_6_0_Stage_8A_Sensor_Safety_V1/V2_Compact.ino and Stage_8B_Suspended_Integration_V1.ino.

atlas_vision_stage12a1_camera1.py and vision dataset collectors.

freeze_edge_roi_release.py, freeze_directional_release.py, resolve_and_freeze_directional_release.py.

atlas_live_dual_model_v1.py and strict-edge live console.

Atlas_6_0_Integrated_RC_V3 through V3.5.2_HEAD_BEHAVIOR_FIX.ino.

atlas_6_0_brain_integration_v1.py and atlas_6_0_full_master_v1.py.

Archived development/chat records and demo-development evidence.

# Appendix C - Version Transition

Atlas 6.0 intentionally stopped short of full SLAM/Nav2 mapping. Its main contribution was a proven real-time mobile-robot base: motor control, encoder feedback, closed-loop distance behavior, ultrasonic safety, Wi-Fi/TCP fail-safe communication, PC-side vision, autonomous obstacle recovery, and reintegrated Atlas personality/expression. Atlas 7.0 can therefore focus on mapping, localization, sensor fusion, and navigation rather than rebuilding these lower-level capabilities from zero.